JP 880 PAGER PROGRAMMING

INSTRUCTIONS

Entering Programming Mode

Step 1		                                              Step 2

| Press and hold down  button. | Insert battery, wait approximately 5 seconds until display is on and shows “F-931-0125,” the “3” will be flashing. |
| --- | --- |

Button Function During Programming Mode

   - Used to move positions between numbers, dashes, and activity switches.

   - Used to change value of figures.

(|) - Used to confirm and complete every step of programming.

Programming Procedure

	ENTRANCE PASSWORD; Screen should Display:		PASS  :  0000

## Use the buttons as described above to enter your 4-digit password followed by ( | ) to enter programming mode (default password: 0000).

	FREQUENCY; Screen should Display:				1. F-931.0125

Use the buttons as described above and modify to your carrier’s frequency.

	 CAP CODE;	Screen should Display:				1:1234567  ON

Use the buttons as described above and modify to subscriber’s cap code.

There are four programmable cap codes. To disable a cap code, press the  button until the activity switch is flashing. Change the “ON (active cap code),” to “ -- (inactive cap code),” by pressing the  button.  Please note that the first cap code can not be disabled.

	BAUD; Screen should Display:					2400

Baud rates can be changed between 512, 1200 or 2400 by pressing the  button

	CHANGING PASSWORD; Screen should Display:		PASS  :  0000

To change your current password enter a new 4-digit number followed by the ( | ) button.  When “pass,” is displayed, programming is accepted and completed.  Press the ( | ) button once more to turn the pager on, and the pager now is ready for use.