JP 900FX PAGER PROGRAMMING

INSTRUCTIONS

Entering Programming Mode

Step 1		                                              Step 2

| Press and hold down  button. | Insert battery, wait approximately 5 seconds until display is on and shows “F-931-0125,” the 3 will be flashing. |
| --- | --- |

Button Function During Programming Mode

   	- Used to move positions between numbers, dashes, and activity switches.

   	- Used to change value of figures.

7 dots - Used to confirm and complete every step of programming (button on right side of pager).

Programming procedure

	FREQUENCY; Screen should Display:		1. F-931.0125

Use the buttons as described above and modify to your carrier’s frequency.

	 CAP CODE;	Screen should Display:	1:0001234567

Use the buttons as described above and modify to subscriber’s cap code.  If there is a cap code with 7 digits, skip the first 3 digits by setting them at zeros, and then enter the seven digit cap code.  There are four programmable cap codes. 1: ----------, only when this is displayed, is the cap code inactive.

	COMPLETED PROGRAMMING VERIFICATION; Screen should Display:	Pass

When “pass”, is displayed, programming is accepted and completed.  Press the (|) button twice to turn the pager on, and the pager now is ready for use.

Example:  Step one	F-929.3875	Step two 1: 0007654321	Step three 2: ----------

Step four 3: ----------	Step five 4: ----------

If a JP900Fx is programmed to the information as shown above it is programmed to operate on: Frequency 929.3875/Cap code 7654321/Flex.  Note: All units are E (any) phase and will operate on A, B, C, D, as well as E phase cap codes.