JP 950 PAGER PROGRAMMING

INSTRUCTIONS

Entering Programming Mode

Step 1		                                              Step 2

| Press and hold down  button. | Insert battery, wait approximately 5 seconds until display is on and shows “F-157-0000,” the “3” will be flashing. |
| --- | --- |

Button Function During Programming Mode

   		- Used to move positions between numbers, dashes, and activity switches.

   		- Used to change value of figures.

7 dots (read) 	- Used to confirm and complete every step of programming (button on right side of pager).

Programming Procedure

	ENTRANCE PASSWORD; Screen should Display:			PASS  :  0000

## Use the buttons as described above to enter your 4-digit password followed by the read button to enter programming mode (default password: 0000).

	FREQUENCY; Screen should Display:					1. F-931.0125

Use the buttons as described above and modify to your carrier’s frequency.

	 CAP CODE;	Screen should Display:				          1:1234567  ON

Use the buttons as described above and modify to subscriber’s cap code.

There are six programmable cap codes. To disable a cap code, press the  button until the activity switch is flashing. Change the “ON (active cap code),” to “ -- (inactive cap code),” by pressing the  button.  Please note that the first cap code can not be disabled.

 	BAUD; Screen should Display:							2400

Baud rates can be changed between 512, 1200 or 2400 by pressing the  button

	CHANGING PASSWORD; Screen should Display:			PASS  :  0000

To change your current password enter a new 4-digit number followed by the read button.  When “pass,” is displayed, programming is accepted and completed.  Press the read button once more to turn the pager on, and the pager now is ready for use.

	COMPLETED PROGRAMMING VERIFICATION; Screen should Display:	Pass

When “pass,” is displayed, programming is accepted and completed.  Press the read button twice to turn the pager on, and the pager now is ready for use.